Computer Network Lab – Week 1

PES1UG20CS806

Divyanshu Sharma

Task 1: Linux Interface Configuration (ifconfig / IP command)

Step 1: To display status of all active network interfaces.

Command Used:  ifconfig (Displaying all the active network interface)

Analyse and fill the following table:

| Interface Name | IPv4/IPv6 | MAC Address |
| --- | --- | --- |
| Enp2s0 | 10.2.20.217 / fe80::9ad5:a2d4:44b4:8a5f | b8:ae:ed:a5:a6:ab |
| Lo | 127.0.0.1 / ::1 | 00:00:00:00:00:00 |

Step 2: To assign an IP address to an interface, use the following command.

Command Used: sudo ifconfig interface_name 10.0.your_section.your_sno netmask 255.255.255.0

: sudo ifconfig enp2s0 10.0.8.06 netmask 255.255.255.0

Step 3: To activate / deactivate a network interface, type.

Command Used:   sudo ifconfig interface_name down

sudo ifconfig enp2s0 down  (Deactivating enp2s0)

Command Used:   sudo ifconfig interface_name up

sudo ifconfig enp2s0 up (Activating enp2s0)

Step 4: To show the current neighbor table in kernel, type

Command Used:  ip neigh

Task 2: Ping PDU (Packet Data Units or Packets) Capture

Step 1: Assign an IP address to the system (Host). Note: IP address of your system should be           10.0.your_section.your_sno.

Step 2: Launch Wireshark and select ‘any’ interface

Step 3: In terminal, type ping 10.0.your_section.your_sno

Command Used – ping 10.0.8.06

Observations to be made

Step 4: Analyse the following in Terminal

• TTL

• Protocol used by ping

• Time

| TTL | 64 |
| --- | --- |
| Protocol used by ping | ICMP |
| Time | Order of 10-2 ms |

Step 5: Analyze the following in Wireshark On Packet List Pane, select the first echo packet on the list. On Packet Details Pane, click on each of the four “+” to expand the information. Analyze the frames with the first echo request and echo reply and complete the table below.

Showing Request and Response Packet

Request Packet

Response Packet

| Details | First Echo Request | First Echo Reply |
| --- | --- | --- |
| Frame Number | 5 | 6 |
| Source IP address | 10.0.8.6 | 10.0.8.6 |
| Destination IP address | 10.0.8.6 | 10.0.8.6 |
| ICMP Type Value | 8 | 0 |
| ICMP Code Value | 0 | 0 |
| Source Ethernet Address | 00:00:00:00:00:00 | 00:00:00:00:00:00 |
| Destination Ethernet Address | 00:00:00:00:00:00 | 00:00:00:00:00:00 |
| Internet Protocol Version | IPv4 | IPv4 |
| Time to Live (TTL) Value | 64 | 64 |

Task 3: HTTP PDU Capture

Using Wireshark’s Filter feature

Step 1: Launch Wireshark and select ‘any’ interface. On the Filter toolbar, type-in ‘http’ and press enter

Step 2: Open Firefox browser, and browse www.flipkart.com

Request Packet

Response Packet

Observations to be made

Step 3

| Details | First Echo Request | First Echo Reply |
| --- | --- | --- |
| Frame Number | 1914 | 1916 |
| Source Port | 53792 | 80 |
| Destination Port | 80 | 53792 |
| Source IP address | 10.2.20.199 | 163.53.78.110 |
| Destination IP address | 163.53.78.110 | 10.2.20.199 |
| Source Ethernet Address | b8:ae:ed:a5:a5:e4 | 14:58:d0:d4:86:00 |
| Destination Ethernet Address | 14:58:d0:d4:86:00 | b8:ae:ed:a5:a5:e4 |

(Connection Details)

Step 4: Analyze the HTTP request and response and complete the table below

| HTTP Request |  | HTTP Response |  |
| --- | --- | --- | --- |
| Get | GET / HTTP1.1\r\n | Server | nginx |
| Host | www.flipkart.com | Content-Type | Text/html |
| User-Agent | Mozilla/5.0 (X11; Ubuntu; Linux x86_64; rv:72.0) Gecko/20100101 Firefox/72.0\r\n | Date | Wed, 27 Jan 2021 07:50:41 GMT |
| Accept-Language | En-US, en;q=0.5\r\n | Location | https://www.flipkart.com/ |
| Accept-Encoding | Gzip, deflate\r\n | Content-Length | 178\r\n |
| Connection | Keep-alive | Connection | Keep-alive |

Task 4: Capturing packets with tcpdump

Step 1: Use the command tcpdump -D to see which interfaces are available for capture.

Command Used -  sudo tcpdump -D

(Viewing Interfaces available for Capture)

Step 2: Capture all packets in any interface by running this command:

Command Used -  sudo tcpdump -i any

(Capturing all Packets in any Interface)

Step 4: To filter packets based on protocol, specifying the protocol in the command line. For example, capture ICMP packets only by using this command:

Command Used - sudo tcpdump -i any -c5 icmp

Step 5: Check the packet content. For example, inspect the HTTP content of a web request like this:

Command Used - sudo tcpdump -i any -c10 -nn -A port 80

Task 5: Perform Traceroute checks

Step 1: Run the traceroute using the following command.

Command Used - sudo traceroute www.google.com

Step 2: Analyze destination address of google.com and no. of hops

The destination address is 142.250.76.68 and there were 30 hops.

Step 3: To speed up the process, you can disable the mapping of IP addresses with hostnames by using the -n option

Command used -  sudo traceroute -n www.google.com

(Disabling mapping of IP addresses with hostnames)

Step 4: The -I option is necessary so that the traceroute uses ICMP.

Command Used - sudo traceroute -I www.google.com

(traceroute with ICMP Protocol)

Step 5: By default, traceroute uses icmp (ping) packets. If you’d rather test a TCP connection to gather data more relevant to web server, you can use the -T flag.

Command Used - sudo traceroute -T www.google.com

(Testing TCP Connection with traceroute)

Task 6: Explore an entire network for information (Nmap)

Step 1: You can scan a host using its host name or IP address, for instance.

Command Used -  nmap www.pes.edu

(Scanning Host with Hostname)

Step 2: Alternatively, use an IP address to scan.

Command Used -  nmap 163.53.78.128

(Scanning Host with IP Address)

Step 3: Scan multiple IP address or subnet (IPv4)

Command Used -  nmap 192.168.1.1 192.168.1.2 192.168.1.3

Task 7 a): Netcat as Chat tool

a) Intra system communication (Using 2 terminals in the same system)

Step 1: Open a terminal (Ctrl+Alt+T). This will act as a Server.

Step 2: Type nc -l any_portnum (For eg., nc -l 1234)

Command Used : nc –l 8090

Note: It will goto listening mode

Step 3: Open another terminal and this will act as a client.

Step 4: Type nc  <your-system-ip-address> portnum

Command Used : nc 10.2.20.199 8090

Note: portnum should be common in both the terminals (for eg., nc 10.0.2.8 1234)

Step 5: Type anything in client will appear in server

Task 7 b): Use Netcat to Transfer Files The netcat utility can also be used to transfer files.

Step 1: At the server side, create an empty file named ‘test.txt’  sudo nc -l 555 > test.txt

Step 2: At the client side, we have a file ‘testfile.txt’. Add some contents to it.

Step 3: Run the client as:  sudo nc 10.0.2.8 555 < testfile.txt here, 10.0.2.8 is the IP address of server and 555 is the port number.

Step 4: At server side, verify the file transfer using the command  cat test.txt

Task 7 c): Other Commands

1) To test if a particular TCP port of a remote host is open.

Command Used :-  nc -vn 10.0.2.8 555

2) Run a web server with a static web page.

Step 1: Run the command below on local host (e.g. 10.0.2.8) to start a web server that  serves test.html on port 80.

Command Used:-  while true; do sudo nc -lp 80 < test1.html; done

Step 2: Now open http://10.0.8.80/test1.html from another host to access it.

Step 3: Observe the details on the terminal

Questions

- Is your browser running HTTP version 1.0 or 1.1? What version of HTTP is the server?

Answer – The Firefox browser used is running HTTP v1.1, and this can be seen in the request header which contains the method (GET) followed by the HTTP version. Similarly, the HTTP version of the web server is v1.1 and can be seen in the header of the HTTP response sent back to the browser.

Request

Response

- How to tell ping to exit after a specified number of ECHO_REQUEST packets?

Answer – Ping continues to send ICMP packages until it receives an interrupt signal. To specify the number of ECHO_REQUEST packages after which ping will exit, we can use the -c option followed by the number of packages. ping -c 10 www.pes.edu