CN LAB – WEEK 6

DIVYANSHU SHARMA

PES1UG20CS806

- Task

  - Configuring Network:

- The network’s is organised in the required topology shown below.

- The IP Addresses for different interfaces have been assigned being used on the routers and systems.

- The routing tables are configured manually by adding the required routing information

- A PDU packet is then transferred from one system on a network to another system on a different network.

- If the packet is transfer successfully, a successful message will be shown in the bottom right corner, otherwise unsuccessful message will be shown.

PC0

PC1

PC2

PC3

ROUTER 0 (FastEthernet0/0)

ROUTER 0 (FastEthernet0/1)

ROUTER 0 (static)

ROUTER 1 (FastEthernet0/0)

ROUTER 1 (FastEthernet0/1)

ROUTER 1 (static)

  - Network and Routing Tables

1.2.1. End Systems

| End System | Interface Name | IP Address | Subnet Mask | Gateway |
| --- | --- | --- | --- | --- |
| PC0 | FastEthernet0 | 10.0.0.1 | 255.0.0.0 | 10.0.0.3 |
| PC1 | FastEthernet0 | 10.0.0.2 | 255.0.0.0 | 10.0.0.3 |
| PC2 | FastEthernet0 | 30.0.0.2 | 255.0.0.0 | 30.0.0.1 |
| PC3 | FastEthernet0 | 30.0.0.3 | 255.0.0.0 | 30.0.0.1 |

1.2.2. Routers

| Router | Interface Name | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| Router0 | FastEthernet0/0 | 10.0.0.3 | 255.0.0.0 |
| Router0 | FastEthernet0/1 | 20.0.0.1 | 255.0.0.0 |
| Router1 | FastEthernet0/0 | 20.0.0.2 | 255.0.0.0 |
| Router1 | FastEthernet0/1 | 30.0.0.1 | 255.0.0.0 |

1.2.3. Routing Tables

| Router | Destination Network | Next Hop |
| --- | --- | --- |
| Router0 | 30.0.0.0 | 20.0.0.2 |
| Router1 | 10.0.0.0 | 20.0.0.1 |

- Task 2

  - Configuring Topology:

The task performed above is repeated with another topology and with different networks and the routing tables are configured accordingly.

PC0 -1.1.1.2/8

PC1 -1.1.1.3/8

PC0

PC2 -2.2.2.2/8

PC3 -2.2.2.3/8

Router0 (FastEthernet0/0)

Router0 (FastEthernet0/1)

Router0 (static)

Router1 (FastEthernet0/0)

Router1 (Ethernet0/0/0)

Router1 (static)

Router2 (FastEthernet0/0)

Router2 (FastEthernet0/1)

Router2 (static)

  - Configuring Network and Routing Tables

    - End Systems

| End System | Interface Name | IP Address | Subnet Mask | Gateway |
| --- | --- | --- | --- | --- |
| PC0 -1.1.1.2/8 | FastEthernet0 | 1.1.1.2 | 255.0.0.0 | 1.1.1.1 |
| PC1 -1.1.1.3/8 | FastEthernet0 | 1.1.1.3 | 255.0.0.0 | 1.1.1.1 |
| PC0 | FastEthernet0 | 6.6.6.2 | 255.0.0.0 | 6.6.6.1 |
| PC2 -2.2.2.2/8 | FastEthernet0 | 2.2.2.2 | 255.0.0.0 | 2.2.2.1 |
| PC3 -2.2.2.3/8 | FastEthernet0 | 2.2.2.3 | 255.0.0.0 | 2.2.2.1 |

    - Routers

| Router | Interface Name | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| Router0 | FastEthernet0/0 | 1.1.1.1 | 255.0.0.0 |
| Router0 | FastEthernet0/1 | 3.3.3.1 | 255.0.0.0 |
| Router1 | FastEthernet0/0 | 3.3.3.2 | 255.0.0.0 |
| Router1 | FastEthernet0/1 | 5.5.5.2 | 255.0.0.0 |
| Router1 | Ethernet0/0/0 | 2.2.2.1 | 255.0.0.0 |
| Router2 | FastEthernet0/0 | 5.5.5.1 | 255.255.255.0 |
| Router2 | FastEthernet0/1 | 6.6.6.1 | 255.255.255.0 |

    - Routing Table

| Router | Destination Network | Next Hop |
| --- | --- | --- |
| Router0 | 2.2.2.0 | 3.3.3.2 |
| Router0 | 5.5.5.0 | 3.3.3.2 |
| Router0 | 6.6.6.0 | 3.3.3.2 |
| Router1 | 1.1.1.0 | 3.3.3.1 |
| Router1 | 6.6.6.0 | 5.5.5.1 |
| Router2 | 3.3.3.0 | 5.5.5.2 |
| Router2 | 1.1.1.0 | 5.5.5.2 |
| Router2 | 2.2.2.0 | 5.5.5.2 |